# Model Brief

## Quick Guide on Wetland BGC

Here we attempt to provide a quick guide to wetland biogeochemistry that also align with the mechanisms of the new development in the model. We focus on the wetland soil since the other components of the wetland are relatively well studied and resolved in the current published model. The soils of wetlands are particularly organic due to the accumulation of biomass and the lack of mineralization of it. This unique feature leads to an abundance of soil organic matter which thus serves as a good starting point of the biogeochemical cycle.

### step one

First, the organic matter in soil got broken down by aerobic microbials through respiration. This respiration is similar to the regular respiration of a plant where it metabolize organic matter into CO2 and water. The soil research community typically measure this as soil respiration, which refers to the total metabolism of all the aerobic microbials combined in a given soil. It is widely accepted that soil respiration is sensitive to temperature and soil moisture (Kirschbaum, 1995; Moyano et al., 2012; Orchard & Cook, 1983). The temperature sensitivity is typically modelled with an exponential function as below:

SR25 is the soil respiration measured at room temperature and Q10 is how much this would change every 10 degree Celsius of temperature change. The above two parameters has been measured for many types of soil and ecosystems. In our work, we took values from a systematic research where temperature and soil moisture were both considered (Meyer et al., 2018). We chose the parameter from grassland or cropland with water content 75% or higher due to their close representation of wetland environment. Currently it is SR25 at 75% water content and Q10 equals 2.

This could be further modified through the Oxygen level in the pore water of soil. Which would then follow a Michealis-Menten Kinetics (Sierra & Renault, 1995). This means the previously calculated soil respiration would be taken as the maximum reaction velocity. An inhibition term is then applied.

CO2KmO2+CO2

As you can tell, this is a fraction valued between 0 and 1 to scale the maximum reaction velocity based on oxygen level. The constant KmO2 here is half saturation rate of oxygen in estuarine water. This is in the current code but disabled for simplicity.

### step two

By mineralizing organic materials, we release not only CO2 and water but also ammonium as N stored in organic matter primarily turned into this form. Ammonium therefore could be written as a product of combined root basil respiration and soil respiration. A conversion was made through C2N ratio (30) in common plant materials as this would be the major source of organic matter in wetland soil.

The next step is nitrification which turns ammonium to nitrate. This was modeled following Michealis-Menten Kinetics as well. This relationship is also called Monod equation and typically takes the form of the following:

where Um is the maxim reaction velocity or enzyme activity and S represents the availability of a specific material/ingredients for the reaction, Ks1 is the half saturation level of that material where the reaction rate is half of the maximum (Shah & Coulman, 1978). When multiple substrates are limiting, the equation could be changed to this:

For now, in our work, Vmax for nitrification is 30 mg/L/day Km_NH4 is 15 mg/L and Km_O2 is 0.8 mg/L (Shah & Coulman, 1978).

### step three

Once we have nitrate and ammonium in the model, denitrification can be coded. There are two kinds of denitrification in wetland environments and they compete for nitrate. Denitrification that turns nitrate to N2 and Dissimilatory Nitrate Reduction to Ammonium (DNRA). We modeled the first again following Michealis-Menten kinetics. For denitrification to N2, Vmax=2.09 mg of N/L/hour and Km_NO3 is 2.6 mg of N/L. For DNRA, the process requires multiple steps/enzymes and therefore is sensitive to multiple stressors that our model can not quite capture in high resolution or time dependent way. Therefore, we took existing literature relationships that DNRA could be described as a fraction of the total denitrification and therefore a linear relationship to the denitrification to N2. We take 0.48 in the model as fraction of DNRA to total denitrification (Li et al., 2019).

### step four

After completing the N cycle, we then calculate the SO4 level in soil so to correctly reflex the methanogenesis. SO4 primarily comes from sea water for estuaries and wetlands and we used a linear relationship to tie SO4 to salinity (Testa et al., 2013):

SO4=20+27190607.445Salinity

The above takes salinity in ppm form and gives SO4 concentration in mg/L form. This code was further improved to maintain SO4 level when dry and only allow sulfate reduction to consume SO4 but decouple the consumption with SO4 concentration when wet.

For sulfate reduction, Vmax=39.3 nmol/cm3/hour and Km_SO4=0.95 nmol/L (Richards & Pallud, 2016).

### step five

Finally for Methanogenesis, two limiting substrates were taken into account (NO3 and SO4)

Vmax is sensitive to temperature and a pre-exponential number (taken from their code) half saturation of the ingredients which is labile carbon pool are taken from (Oikawa et al., 2024). Two more inhibition term were used since the presence of those two materials will slow down the methane production.

## Parameters:

| Parameters | Value |
| --- | --- |
| labile/refractory ratio according to organic percentages(Unger et al., 2016) |  |
| SR25: Soil respiration rate at 25 degree Celsius with 75% soil moisture | 2.68 ug of CO2 per hour |
| Q10: Soil respiration temperature sensitivity | 2 |
| KmO2: Half saturation concentration for O2 | 2590 mmol/m3 |
| Km_NH4 for nitrification | 15 mg/L |
| Km_O2 for nitrification | 0.8 mg/L |
| Vmax for regular denitrification | 2.09 mg of N/L/hour |
| DNRA/Total Denitrification | 0.48 |
| Km_NO3: Half saturation rate of NO3 for regular denitrification | 2.6 mg of N/L |
| Vmax for nitrification | 30 mg/L/day |
| Vmax for sulfate reduction | 39.3 nmol/cm3/hour |
| Km_SO4 for sulfate reduction=nmol/L | 0.95 nmol/L |
| marsh uptake of N partition between nh4 and no3 (modeled like phytoplankton) Pg:132 Eq(4.2.12) |  |
| ai: pre exponential factor for methanogenesis(Oikawa et al., 2024) | 6.21014 |
| Eai: activation energy of the enzymatic reaction with the substrate | 86.9 |
| R: universal gas constant | 8.314 |
| Kmlabile: Half saturation rate of soil organic carbon for methanogenesis | 23.5 g of C/m3 |
| KINO3: Inhibition coefficient of NO3 for methanogenesis | 0.1 mg/L |
| KISO4: Inhibition coefficient of SO4 for methanogenesis | 486.41 mg/L |
| so4 to salinity (from Jeremy) | 27/190607.455 from ppm to mg/L |

References:

Kirschbaum, M. U. F. (1995). The temperature dependence of soil organic matter decomposition, and the effect of global warming on soil organic C storage. Soil Biology and Biochemistry, 27(6), 753–760. https://doi.org/10.1016/0038-0717(94)00242-S

Li, X., Gao, D., Hou, L., & Liu, M. (2019). Soil substrates rather than gene abundance dominate DNRA capacity in the Spartina alterniflora ecotones of estuarine and intertidal wetlands. Plant and Soil, 436(1–2), 123–140. https://doi.org/10.1007/s11104-018-03914-w

Meyer, N., Welp, G., & Amelung, W. (2018). The Temperature Sensitivity (Q10) of Soil Respiration: Controlling Factors and Spatial Prediction at Regional Scale Based on Environmental Soil Classes. Global Biogeochemical Cycles, 32(2), 306–323. https://doi.org/10.1002/2017GB005644

Moyano, F. E., Vasilyeva, N., Bouckaert, L., Cook, F., Craine, J., Curiel Yuste, J., Don, A., Epron, D., Formanek, P., Franzluebbers, A., Ilstedt, U., Kätterer, T., Orchard, V., Reichstein, M., Rey, A., Ruamps, L., Subke, J.-A., Thomsen, I. K., & Chenu, C. (2012). The moisture response of soil heterotrophic respiration: interaction with soil properties. Biogeosciences, 9(3), 1173–1182. https://doi.org/10.5194/bg-9-1173-2012

Oikawa, P. Y., Sihi, D., Forbrich, I., Fluet‐Chouinard, E., Najarro, M., Thomas, O., Shahan, J., Arias‐Ortiz, A., Russell, S., Knox, S. H., McNicol, G., Wolfe, J., Windham‐Myers, L., Stuart‐Haentjens, E., Bridgham, S. D., Needelman, B., Vargas, R., Schäfer, K., Ward, E. J., … Holmquist, J. (2024). A New Coupled Biogeochemical Modeling Approach Provides Accurate Predictions of Methane and Carbon Dioxide Fluxes Across Diverse Tidal Wetlands. Journal of Geophysical Research: Biogeosciences, 129(10). https://doi.org/10.1029/2023JG007943

Orchard, V. A., & Cook, F. J. (1983). Relationship between soil respiration and soil moisture. Soil Biology and Biochemistry, 15(4), 447–453. https://doi.org/10.1016/0038-0717(83)90010-X

Richards, C. M., & Pallud, C. (2016). Kinetics of sulfate reduction and sulfide precipitation rates in sediments of a bar-built estuary (Pescadero, California). Water Research, 94, 86–102. https://doi.org/10.1016/j.watres.2016.01.044

Shah, D. B., & Coulman, G. A. (1978). Kinetics of nitrification and denitrification reactions. Biotechnology and Bioengineering, 20(1), 43–72. https://doi.org/10.1002/bit.260200105

Sierra, J., & Renault, P. (1995). Oxygen consumption by soil microorganisms as affected by oxygen and carbon dioxide levels. Applied Soil Ecology, 2(3), 175–184. https://doi.org/10.1016/0929-1393(95)00051-L

Testa, J. M., Brady, D. C., Di Toro, D. M., Boynton, W. R., Cornwell, J. C., & Kemp, W. M. (2013). Sediment flux modeling: Simulating nitrogen, phosphorus, and silica cycles. Estuarine, Coastal and Shelf Science, 131, 245–263. https://doi.org/10.1016/j.ecss.2013.06.014

Unger, V., Elsey-Quirk, T., Sommerfield, C., & Velinsky, D. (2016). Stability of organic carbon accumulating in Spartina alterniflora-dominated salt marshes of the Mid-Atlantic U.S. Estuarine, Coastal and Shelf Science, 182, 179–189. https://doi.org/10.1016/j.ecss.2016.10.001